MDA691 — Short Project Note

Real-Time Data with Agentic AI for Industrial Predictive Maintenance (PdM)

Team: Amit Shrestha, Subala Shrestha, Asmita Pandey, Teegala Pavan Reddy  ·  Client: Miroslav Kostecki (eLabtronics/runlinc)

1. What we are doing

We are building a laboratory prototype on client-supplied STEMSEL E32W (ESP32) hardware with the 37-in-1 sensor kit, programmed through runlinc IDE (MicroPython). The system uses a dual-path design: (1) fast edge reflex — vibration/temperature features, ISO 10816-3 local alarm, and buzzer (< 200 ms, offline capable); and (2) slower analytics — MQTT streaming, LSTM RUL prediction, runlinc AI Assistant for maintenance recommendations, Thought Log dashboard, and simulated ERP actions. Primary kit modules: Shock (KY-002), TEMP 18B20, buzzer. Assignment 2 delivers the project plan and intermediate design; full implementation is scheduled for Weeks 9–20.

2. Challenges we are facing

Lab data may not provide enough failure/degradation labels for RUL — mitigated with C-MAPSS/XJTU-SY benchmarks.

E32W hotspot and device connection issues (observed 17 August 2026).

ESP32 memory limits may restrict FFT bands and on-device model size.

Team bench data may not fully represent factory operating conditions.

MQTT/agent pipeline could fail at demo — local reflex alarm must continue independently (replay harness).

Agent may produce invalid or inconsistent tool calls — JSON schema validation and human review required.

Saving and retrieving sensor data reliably across runlinc sessions (raised 7 September 2026).

3. Client meetings and what the client suggested

3.1 Earlier meetings (August 2026)

| Date | Focus | Client direction / outcome |
| --- | --- | --- |
| 10 Aug 2026 | Project kick-off | Document review; meeting schedule agreed |
| 17 Aug 2026 | runlinc setup | Hotspot configuration; device connection troubleshooting; runlinc login |
| 24 Aug 2026 | Hardware lab | LED blink (D2); runlinc Projects 14/15; block diagram planning; DS18B20 test later (23.125 °C) |
| Ongoing | Data scope | Team collects all real-time data from supplied E32W + kit — client factory/plant data out of scope |

3.2 Meeting — 7 September 2026 (Monday)

Attendees: Miroslav Kostecki (client/industrial supervisor); Amit Shrestha; Teegala Pavan Reddy; Asmita Pandey; Subala Shrestha.

Discussion points and client guidance:

Project introduction: team explained objectives and work completed so far to the supervisor and a guest colleague.

Sensor suggestions: client advised using suitable sensors to monitor temperature and other relevant conditions; discussed specifications and possible solutions for PdM.

Documentation and presentation: guidance on organising the project report, submission requirements, and how to present the work.

Block diagram: discussed how the diagram should show kit sensors, E32W connections, data processing, and system components — aligned with our Figure 1 architecture plan.

Lifespan / predictive maintenance: discussed detecting or estimating equipment remaining useful life (RUL) and when maintenance is needed from sensor data.

Code persistence: discussed how to send, retrieve, and reuse runlinc code so previous progress is not lost between sessions.

Runlinc Intermediate Project 15: client advised reviewing Project 15 to learn how to save sensor data and retrieve it as a CSV file.

Agreed next steps (from 7 Sep meeting):

Review Runlinc Intermediate Project 15 (CSV save/retrieve).

Continue improving the block diagram with correct sensor names and data flow.

Investigate suitable sensors and parameters for predictive maintenance.

Proceed with wiring Shock (GPIO 23), buzzer (GPIO 34), and live data pipeline toward MQTT and agent integration.

4. Summary

The client supports a lab-based prototype on supplied E32W hardware, with the team owning all data collection. Key technical direction from meetings: use runlinc intermediate projects (14 for temperature, 15 for CSV export), build a clear block diagram, focus on lifespan/RUL from sensor streams, and keep fast edge alarms separate from slower agentic AI. Main risks remain connectivity, limited lab failure labels, ESP32 constraints, and reliable data/code persistence across runlinc sessions.

Prepared from: MDA691_Assignment2_Group_Report_SUBMISSION.docx and Meeting Minutes 7 September 2026.